The Commonwealth of Massachusetts

Executive Office of Health and Human Services

Department of Public Health

Bureau of Environmental Health

Food Protection Program

Mobile Poultry Processing Unit

Food and Farm Safety Management Guide

for Small-Scale Poultry Producers and Processors

Using a Massachusetts-Inspected MPPU

Adopted from the New England Small Farm Institute

and the New Entry Sustainable Farming Project

Formatted: May 2011

by the

Food Protection Program

Bureau of Environmental Health

Massachusetts Department of Public Health

Table of Content

Preface	4

Chapter 1 - MPPU Safe Food-Handling Plan

1.0 - Introduction	5

1.1 - MPPU Processed Food Products	6

1.2 - MPPU Operation	7

1.3 - MPPU Food Production Process Flow Diagram	8

Chapter 2 - Good Manufacturing Practices

2.0 - Introduction	9

2.1 - Provide Training for Processing Personnel	9

2.2 - Establish Health and Hygiene Policies for Processing Personnel	9

2.3 - Create and Maintain a Clean Processing Environment	10

2.4 - Control Pests: Inside and Outside	11

2.5 - Control Access	11

2.6 - Provide and Protect Potable Water	11

2.7 - Maintain and Securely Store Processing Equipment and Utensils	12

2.8 - Provide Secure Storage for Processing Supplies and Materials	12

2.9 - Manage Processing Wastes	12

Chapter 3 - Standard Operating Procedures

3.0 - Introduction	14

3.1 - SOP for Site Management and Pest Control	14

3.2 - SSOP for Personnel Health and Hygiene	14

3.3 - SSOP: Pre-Operational Inspection and Sanitation Schedule	14

3.4 - SSOP: Daily Operational Sanitation Maintenance	15

3.5 - SSOP for Chill Tank and Refrigeration Temperature Monitoring	15

3.6 - SSOP: Post-Operational Sanitation Schedule	16

Chapter 4 - Hazard Analysis Critical Control Point Plan

4.0 - Introduction	17

4.1 - The Seven Steps of HACCP	17

4.2 - Hazard Analysis and Identification of Critical Control Points	19

4.3 - Identifying Critical Limits, Monitoring and Corrective Actions	23

4.3 - Identifying Critical Limits, Monitoring and Corrective Actions	26

4.4 - HACCP Record Keeping and Verification Procedures	27

Appendices

(A) Sample Operation Recordkeeping Logs………….………………………………….28

(B) Sample Farm-to-Farm Bio-Security Protocol and Practices……  …………….……43

(C) Sample MPPU Processing Water and Solid Waste Management Protocol and Practices Log………….……………………………...…………………                    45

(D) DEP Guidelines and Best Management Practices for MPPU Waste Management………….47

Preface

Throughout the United States, farmers and service providers are exploring the use of “mobile poultry processing units” (MPPUs) – a processing option that lets small-scale producers (those raising and processing more than 1,000 but fewer than 20,000 chickens or 5,000 turkeys each year) process poultry on their own farms, and market their products in their own States.

The Mobile Poultry Processing Unit, Food and Farm Safety Management Guide for Small-Scale Poultry Producers and Processors Using a Massachusetts-Inspected MPPU is intended to inform and provide guidance to small-scale producers in the safe use of a Massachusetts-approved MPPU.

Chapter 1

MPPU Safe Food-Handling Plan

1.0 - Introduction

The purpose of the MPPU Safe Food-Handling Plan is to insure that the products – whole raw poultry and giblets – offered for sale by Massachusetts’ smallest-scale poultry producers using a Massachusetts-approved MPPU – are wholesome and processed under clean and sanitary conditions, and that the operation meets Massachusetts Department of Environmental Protection (DEP) guidelines for waste disposal, and, therefore, does not contribute to environmental harm.

The MPPU Safe Food-Handling Plan begins with a clear description of the specific foods to be produced (whole raw poultry and giblets) and a flow chart that includes each step of the food production process. These are followed by:

- Good Manufacturing Practices (GMPs) that describe proper practices for safe and sanitary handling of foods,

- Standard Operating Procedures (SOPs),

- Sanitation Standard Operating Procedures (SSOPs) that describe the actual steps undertaken each day to insure sanitary food handling and general hygiene practices (and how to perform activities that ensure sanitary food handling and facility cleanliness), and

- Hazard Analysis Critical Control Point (HACCP) – a food safety management system that helps processors identify and control food safety hazards in their operations. While HACCP is widely regarded as the heart of a Safe Food Handling Plan, it is built on the foundation of carefully considered GMPs and SOPs.

|  | 3. HACCP |  |
| --- | --- | --- |
| 1. Good Manufacturing Practices |  | 2. Sanitation Standard Operating Procedures |

HACCP is not a stand-alone program

1.1 – MPPU-Processed Food Products

Common Name: 		Fresh, whole raw poultry and giblets.

Uses: 	To be cooked by consumers.

### Packaging: 			Plastic bags or other approved method.

How Sold: 	Fresh or frozen (if fresh, picked up within four hours of slaughter by consumer or held at <41F for no more than four days).

Where Processed: 	Processed on producer’s own farm, using a Massachusetts MPPU.

Where/To 	Product may be offered for sale at the producer’s own farm or

Whom Sold:	farm stand and at other approved locations, directly to “household consumers, restaurants, hotels or boarding houses that purchase whole raw poultry for use in their own dining rooms or in the preparation of meals for sale direct to consumers.” (Reference: Poultry Processing Inspection Act §464 (c) 1 (d).)

Labeling 			Labels must include:

          - producer/processor’s name and address,

          - slaughter date,

          - “Exempt P.L. 90-492,” and

          - safe handling instructions. The graphic below is available from http://www.fsis.usda.gov/images/KC/safe_handling_label_hi.jpg

1.2 - MPPU Operation

### WATER USED IN PROCESSING and SANITATION:

All water used on the MPPU for processing, cleaning and sanitation, chilling tanks and ice manufacture, and used in the production of whole raw poultry carcasses and giblets, must be potable.

CLEANING and SANITATION AGENTS:

All cleaning agents (soaps and detergents) must be biodegradable and used in prescribed label concentrations and methods.

Approved sanitizing agents for use on food contact surfaces must be biodegradable and used in prescribed concentrations and methods.

Including: Chlorine (i.e., household bleach), Hydrogen peroxide, and quaternary ammonia-based sanitizers.

EQUIPMENT MAINTENANCE AGENTS:

Any agents used in equipment maintenance (whether such equipment is located on the MPPU or used in the production of whole raw poultry carcasses and giblets), including any lubricants applied to equipment surfaces subject to corrosion after final cleaning, rinse and sanitation, must be food grade.

ENVIRONMENTAL CONSIDERATIONS:

The MPPU unit must be managed in a manner that protects the environment and is in accordance with Massachusetts Department of Environmental Protection guidelines, including those related to wastes and groundwater. Please refer to:

- MPPU Good Manufacturing Practices #2.9, page 12,

- MPPU Processing Water and Solid Waste Management Protocol and Practice Log - Appendix C, page 45, and

- DEP Guidelines and Best Management Practices for MPPU Waste Management - Appendix D, page 47.

1.3 - MPPU Food Production Process Flow Diagram

Chapter 2

Good Manufacturing Practices

2.0 - Introduction

The following MPPU Good Manufacturing Practices (GMPs) describe what procedures must be followed in order to process safe and wholesome poultry. Approved Massachusetts-inspected MPPU models are designed to support sanitary processing conditions on the unit. The full processing environment extends beyond the unit to include the farm environment. It includes the people, buildings, grounds, equipment and conditions on the farm site. The following GMPs are designed to help create a processing environment that can meet stringent regulatory requirements for the safe and sanitary processing of raw poultry.

2.1 - Provide Training for Processing Personnel

Personnel must be trained to work on the MPPU. Supervisors must be sure that all personnel working on an MPPU unit are trained. Supervisors must keep records of training, which includes information about demonstrations and opportunities for supervised practice, and documented proficiency in the following areas:

- Personal health assessment and hygiene practices (GMP 2).

- Safe and humane poultry processing and handling.

- Proper cleaning and sanitizing protocols (GMP 3-8).

- Safe and effective processing waste management (GMP 9).

- All aspects of SOP/SSOP HACCP program implementation, including recordkeeping.

2.2 - Establish Health and Hygiene Policies for Processing Personnel

Supervisors are required to make certain that personnel have the knowledge, skills and attitude necessary to keep poultry products free from contamination by food handlers. This is especially important because poultry products support the rapid growth of microorganisms and are recognized as a “potentially hazardous food.” Adopt written Personnel Health and Hygiene Policies and provide mandatory training in personal health and hygiene practices before you and your personnel begin to process poultry. Consider attending a ServSafe® or similar food safety training program to insure that you are well informed about safe food handling.

Your Personnel Health and Hygiene Policies and training program must address:

- Personal Health. Personnel should be dismissed from the processing environment if they:

    - Have a food borne illness.

    - Show symptoms of a stomach or intestinal illness or jaundice.

    - Have a sore throat or temperature.

    - Have an infected wound or cut.

    - Live with or are exposed to a person who is ill.

- Personal Cleanliness. Personnel must understand the critical importance of general personal cleanliness. Personnel should report to the processing date (location) clean and wearing clean outer garments, including showering and shampoo before work.

- Hygienic Hand Practices. Hand washing is the most important aspect of personal hygiene for food handlers. Train personnel to follow these steps:

  - Wet hands with running water as hot as you can comfortably stand it (at least 112°F) and apply soap.

  - Vigorously scrub hands and arms for at least ten to fifteen seconds. Pay special attention to cleaning between fingers and under fingernails.

  - Rinse thoroughly under hot running water.

  - Dry hands with a single-use paper towel.

  - Use a paper towel to turn off the faucet and to open the bathroom facility door.

  - Wash hands frequently when handling live or processed poultry or viscera, as well as before starting work, and after:

  - Using toilet facilities.

  - Handling processing by-products or trash.

  - Touching hair, face or body, including an open sore.

  - Sneezing, coughing or using a tissue.

  - Handling chemicals that might affect food safety.

  - Touching dirty clothing, work aprons, work surfaces or anything else that could contaminate hands, such as unsanitary equipment, work surfaces or cleaning tools.

  - Gloves, if used, should be disposable and changed when they become soiled or torn, before changing tasks, and at least every four hours during continued use. Hand dips are optional but not required. Nail polish and the use of fake nails are prohibited; nails should be clipped short.

- Proper Work Attire. Processing personnel must:

    - Wear clean clothing. If possible, change into clean clothes at the processing site.

    - Wear a clean hat or other hair restraint. Personnel with long beards should wear beard restraints.

    - Remove jewelry from hands and arms. Jewelry provides a good host site for pathogens and may pose a hazard when working around equipment.

    - Wear appropriate, clean boots or close-toed shoes with non-skid soles. Consider providing step-in shoe sanitizing “stations” at points of entry to the MPPU.

2.3 - Create and Maintain a Clean Processing Environment

Establish grounds and building maintenance practices that provide a clean and wholesome processing environment.

- Set up or arrange your site, including the location of the MPPU, to allow easy and direct movement of live birds to the holding area. Also provide easy and direct movement of chilled, packaged carcasses from the MPPU to your on-site refrigerated storage areas.

- Maintain the following areas in a clean, well-drained condition and free of litter:

    - Poultry-holding facilities and adjacent areas.

    - The MPPU location (including water and electric hook up).

    - Buildings or sheds used for: storage of processing/handling supplies, equipment, and finished product (i.e., refrigeration or freezing, and adjacent areas).

    - Facilities used by personnel for personal hygiene (i.e., toilets, hand-washing, supplies and clothing) and adjacent areas.

    - On-site areas used for processing waste management (i.e., fields or pastures used for wastewater disposal and compost areas used to process solid wastes).

- Frequently inspect all outside areas of your site for trash, blood, feathers, fecal material, etc., all of which must be promptly and properly removed and disposed.

- Keep trash cans tightly covered.

- Maintain adequate dust control throughout your site.

- Keep the buildings and sheds you use for storing processing supplies and product, and for maintaining personal hygiene of your personnel, in good, easily cleanable repair.

2.4 - Control Pests: Inside and Outside

Install and maintain adequate pest control measures throughout your processing environment.

- Keep all areas free of harborages for rodents and maintain clean zones in and around all storage and processing areas.

- Install measures to prevent wild birds, domestic and wild animals, and insects from entering your processing environment.

- Prevent wild birds and other pests from nesting in the processing environment.

- Inspect all areas at least monthly for presence of rodents and all other pests.

E.	Establish and maintain rigorous on-farm and farm-to-farm bio-security policies and           practices.

2.5 - Control Access

Place signs around your site to provide strict access control in your processing environment.

- Discourage non-personnel from entering your poultry-rearing areas (a bio-security issue)          and processing environment in general, and do not permit them on the MPPU when in use.

- Limit access to poultry-holding areas, the MPPU, and on-site storage/refrigeration areas to trained personnel during processing operations. Personnel must not move back and forth between the MPPU’s slaughter and evisceration areas, between the unit and poultry holding and on-farm refrigeration/storage areas, or out of and back into the processing environment without removing gloves and aprons when leaving, and without washing hands upon return.

- Prohibit smoking, eating, drinking, and chewing gum and tobacco in the processing environment when processing is taking place.

2.6 - Provide and Protect Potable Water

Provide a supply of safe-to-drink, potable water that is sufficient (quantity and pressure) to support all processing, chilling, cleaning, sanitizing and personnel hygiene needs, including ice manufacture. (Sources of potable water include municipal water, private wells that are properly managed and regularly tested, closed portable water containers filled with potable water and bottled drinking water.) In addition:

- Provide hot water (112°F minimum) for personal hygiene (including hand washing) and equipment cleaning.

- Provide approved, food-grade quality hoses and pipes for all water used for processing, cleaning, and personal hygiene.

- Install and maintain measures to prevent contamination of water used in processing, cleaning, and personal hygiene; prevent cross-contamination between potable and non-potable water with water system backflow prevention devices (air gaps, vacuum/pressure breakers or check valves).

2.7 - Maintain and Securely Store Processing Equipment and Utensils

Maintain the processing equipment and utensils in good condition so that they can perform effectively and can be easily cleaned and sanitized. Store them securely when not in use.

- Conduct pre- and post-operation inspections of all processing equipment and utensils, checking for cleanliness and signs of rust, wear, damage or other defects. The equipment inspection checklist should include:

- Killing cones

- Scalder and plucker

- Knives and other implements and utensils

- Evisceration and work tables

- Chilling and holding tanks, ice containers, processing waste collection tubs

- Cleaning and sanitizing equipment

- Hoses, water and propane lines and connections, water backflow devices, electric outlets and wiring, propane tanks, etc.

- Repair serious defects and/or perform necessary maintenance before processing begins and prior to storage.

- Store all cleaned and sanitized equipment and utensils in good conditions in clean, secure storage areas, to prevent damage or contamination of any kind.

2.8 - Provide Secure Storage for Processing Supplies and Materials

Store all supplies and materials used in cleaning, sanitizing, packaging and labeling in clean, secure storage areas, to prevent damage or contamination of any kind. Keep cleaning and sanitizing agents in clearly labeled, secure containers, and keep separated from supplies that may come in contact with food.

2.9 - Manage Processing Wastes

Before bringing a Massachusetts-inspected MPPU onto the farm, the producer will be asked to prepare a Processing Wastewater and Solid Waste Management Plan and receive a site inspection from Massachusetts Department of Agricultural Resources (DAR) staff. The plan should describe the steps taken to manage processing wastes in a safe and environmentally responsible manner. The plan will insure that:

- Wastewater, such as water from chilling, cleaning with approved soaps, and rinsing, is properly collected and land applied on biologically active farm hayfields or pastures in a manner that precludes erosion and functions as a safe and appropriate crop nutrient. Such fields or pastures must be located at least 100 feet from any surface water or wells.

- Solid processing waste, such as poultry feathers, blood and viscera, is properly collected, transported and incorporated into an actively managed agricultural compost pile or windrow. The proposed compost “recipe” must support active composting, including appropriate bulking materials, moisture content and C:N ratio.

- Trash, such as discarded containers for supplies, damaged packaging materials and disposable gloves, is properly collected, contained and removed from the processing environment.

Chapter 3

Standard Operating Procedures

3.0 - Introduction

Standard Operating Procedures (SOPs) and Sanitation Standard Sanitation Operating Procedures (SSOPs) are designed to prevent the creation of unsanitary processing conditions and insure that food products are wholesome and unadulterated. They describe how to carry out and document safe food handling and personal hygiene practices (GMPs).

3.1 - SOP for Site Management and Pest Control

- Frequency: monthly throughout the processing season.

- Person responsible: Producer-processor or designee.

- Procedure (see GMPs 3-8):

- Visually inspect processing environment (grounds and buildings, including storage areas and sanitary facilities) for cleanliness and presence of pests. List needed corrective actions.

- Perform corrective actions.

- Document, sign and date in MPPU Operations Log.

3.2 - SSOP for Personnel Health and Hygiene

- Frequency: each day of processing operation.

- Person responsible: Producer-processor or designee.

- Procedure (see GMP 1 and 2):

- Interview and visually check processing personnel for health and personal hygiene considerations, prior to approving anyone for food handling. Dismiss anyone found unsuitable for work.

- Document, sign and date in MPPU Operations Log.

3.3 - SSOP: Pre-Operational Inspection and Sanitation Schedule

- Frequency: each day, prior to processing operation.

- Persons responsible: Producer-processor or designee.

- Procedure:

- Visually inspect all equipment and utensils for cleanliness and operability.

- Clean, rinse and sanitize all product contact surfaces, equipment and utensils, including coolers. If appropriate, test food contact areas for sanitizer residue.

- Document, sign and date in MPPU Operations Log.

3.4 - SSOP: Daily Operational Sanitation Maintenance

- Frequency: each day, throughout processing operation.

- Person responsible: Producer-processor or designee.

- Procedure:

- Kill Area

- If a carcass falls to the floor, pick it up immediately, and wash thoroughly before further processing and document.

- If a piece of equipment or a utensil falls to the floor, wash thoroughly and document.

- Maintain area in a clean and sanitary condition throughout operation.

- Processing Area

- If a carcass or giblet falls to the floor, pick it up immediately, and wash thoroughly before further processing, and document event.

- If intestines are nicked during evisceration, thoroughly wash and sanitize all areas and utensils contaminated with fecal matter, and document event.

- If a piece of equipment or a utensil falls to the floor, wash it thoroughly, and document event.

- Maintain entire area in a clean and sanitary condition throughout the daily operation

- Document required corrective actions, sign and date as required in MPPU Operations Log when daily operation is complete.

3.5 - SSOP for Chill Tank and Refrigeration Temperature Monitoring

- Frequency: test chill tank slurry temperatures once per hour of processing operation, and test and record refrigerator temperature once per day.

- Person responsible: Producer-processor or designee.

- Procedure:

  - Use a digital thermometer to test chill tank ice slurry temperatures. The target temperature for chill tank slurry is between 33°F and 40F. Add ice as necessary to adjust temperature. Document, sign and date as required in MPPU Operations Log.

  - NOTE: The chill tank must reduce the temperature of carcasses to 40F or less within 4 hours of evisceration. See HACCP Critical Control Point #2. Use a digital thermometer to measure internal carcass temperatures of 2% (or a minimum of 5) of poultry. Document, sign and date as required in MPPU Operations Log.

  - Use a digital thermometer to test pre-chill tank water. Add cold water frequently to maintain as cool as possible. Ice water slurry is not required.

  - Use a max-min thermometer to measure refrigerator storage temperatures.  Document, sign and date as required in MPPU Operations Log.

  - NOTE: Hold fresh product at 33-40F until delivery. Stored at these temperatures, product shelf life is four days. Freeze or discard product if held for more than four days.

  - Maintain Farmer’s Market cooler temperatures at 33-40F for fresh product. Record cooler temperatures at start and end of day. Document, sign and date as required in the MPPU Daily Operations Log.

3.6 - SSOP: Post-Operational Sanitation Schedule

      - Frequency: each day, after processing operation.

      - Person responsible: Producer-processor or designee.

      - Procedure:

- Kill Area

  - Pick up feathers and other matter, and deposit into receptacle for inedible material.

  - Briefly pre-rinse all dirty areas with warm water, and start the process at the top and work all material down to the floor.

  - Apply detergent as directed.

      - Rinse all equipment from top to bottom.

      - Inspect and re-clean any missed areas.

      - After cleaning/rinsing work areas, apply sanitizer to all contact surfaces.

      - Squeegee standing water to the floor.

- Processing Area

(1) Pick up any pieces of bones, fat, meat or other matter and deposit into container for inedible material.

(2) Disassemble all equipment and place parts in their designated tubs.

(3) Briefly pre-rinse all soiled areas with warm water. Start the process at the top and work all material down to the floor.

(4) Apply approved soap as directed.

(5) Rinse all equipment from top to bottom.

  - Inspect and re-clean any missed areas.

  - After equipment and work areas have been cleaned, apply sanitizer to all contact surfaces.

  - Squeegee any standing water on floor to drainage areas.

  - Remove, clean and sanitize any waste conduits or drains.

(10) Apply edible oil to all surfaces that are subject to corrosion.

- Document, sign and date as required in MPPU Operations Log.

# Chapter 4

Hazard Analysis Critical Control Point Plan

4.0 - Introduction

The adoption of a Hazard Analysis Critical Control Point (HACCP) plan for poultry is a valuable tool that can assist producers to produce a safer food product. Food safety is a critical concern for food business and for consumers. The failure to control a food safety hazard in your operation can cause food borne illness and result in undesirable legal and economic consequences for producers and the industry.

A HACCP system is a food safety system that helps processors identify and control their operation from the time they receive raw materials and ingredients until they distribute their final product. It focuses on thinking about and eliminating, minimizing, or reducing food safety hazards to an acceptable level. A HACCP program will reduce the likelihood that your operation will produce an unwholesome food – and result in fewer economic losses due to disposal of unsafe foods.

4.1 - The Seven Steps of HACCP

- Assess food safety hazards associated with all areas of your product and your process, and describe measures that prevent the hazards.

- Determine the Critical Control Points (CCPs) – observable and measurable.

- Establish the Critical Limits (standards) for each CCP.

- Establish Monitoring Procedures for the CCPs.

- Establish Corrective Actions to be taken when CCPs are not in control.

- Establish Record-Keeping Procedures that effectively document the HACCP system.

- Establish Verification Procedures to determine that the system is working.

For the seven-step HACCP system to work, the producer must have a thorough knowledge and understanding of the process and the product. Beginning with a “hazard analysis” (Step 1), the processor identifies and reviews all potential food safety problems in the production, processing, packaging and distribution of the product. As a result of this analysis, producers target those points in the process that must be controlled to prevent the development or minimize the effects of a food safety hazard. These points in the process are called Critical Control Points (Step 2). Critical Limits are set for each Critical Control Point or CCP (Step 3). The Critical Limits at each CCP are monitored (Step 4) and Corrective Actions (Step 5) are taken if the system is not in control and a food safety hazard exists.

A HACCP plan includes a record-keeping system (Step 6) that validates that the potential food safety hazards in the process are under control. The producer will need a plan to regularly review the records and the process to verify that the HACCP system is performing adequately (Step 7).

Each HACCP plan is unique to a specific food product and processing facility. The plan included in the following pages has been developed for use by Massachusetts’ “smallest-scale” poultry producer processors using a Massachusetts-licensed MPPU to produce whole raw poultry carcasses and giblets for direct-to-consumer sale.

4.2 - Hazard Analysis and Identification of Critical Control Points

| Process Step | Potential Hazard Note: “X-C” = cross contamination | What control measures can be applied to prevent the hazard? | Is the potential safety hazard significant and reasonably likely to occur? | CCP# |
| --- | --- | --- | --- | --- |
| Receive/ Hold | Biological: fecal contamination (salmonella ssp.) from birds or infected personnel. | Withhold feed, provide acidified water prior day. Clean any foreign matter from birds. Prevent X-C. Proper personnel hygiene (GMP 2; SSOP 2). | YES. Steps to control contamination occur throughout processing process. |  |
|  | Physical: fecal | See above. | See above. |  |
|  | Chemical: none |  |  |  |
| Kill/Bleed | Biological: pathogen introduction (X-C) | Proper cleaning of cones, equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Scald | Biological: pathogen introduction (X-C) | Monitor water temperature; change water if/as required. | NO |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Pluck | Biological: pathogen introduction (X-C) | Proper cleaning of equipment, including rubber picker fingers (SSOP 3). | NO |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Pre-Chill | Biological: pathogen introduction (X-C) | Monitor water temperature; change frequently (SSOP 5). | NO |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |

| Process Step | Potential Hazard | Control measures that can be applied to prevent the hazard | Is the potential safety hazard significant and reasonably likely to occur? | CCP # |
| --- | --- | --- | --- | --- |
| Transfer | Biological: none |  |  |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Remove Head/Feet | Biological: accidental X-C | Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Remove Oil Gland | Biological: pathogen introduction (X-C) | Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Make J-Cut around Vent | Biological: accidental fecal contamination (salmonella ssp.) | Proper personnel training (GMP 1); proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Eviscerate | Biological: pathogen introduction | Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |
|  | Physical: none Chemical: none |  |  |  |

| Process Step | Potential Hazard | What control measures can be applied to prevent the hazard? | Is the potential safety hazard significant and reasonably likely to occur? | CCP # |
| --- | --- | --- | --- | --- |
| Inspect Viscera | Biological: none |  |  |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Harvest Liver/ Heart | Biological: pathogen introduction | Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |
|  | Physical: none |  |  |  |
|  | Chemical: none |  |  |  |
| Harvest Neck | Biological: pathogen introduction | Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |
|  | Physical: none Chemical: none |  |  |  |
| Remove Crop and Lungs | Biological: pathogen introduction | Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |
|  | Physical: none Chemical: none |  |  |  |
| Trim Carcass/Final Rinse (Inside and Outside) | Biological: pathogen Introduction Physical: none Chemical: none | Trim to remove any foreign matter that may cause contamination. Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |

| Process Step | Potential Hazard |  | What control measures can be applied to prevent the hazard? | Is potential safety hazard significant and reasonably likely to occur? |  | CCP # |
| --- | --- | --- | --- | --- | --- | --- |
| Final Inspection: Carcass, Giblets and Neck | Biological: pathogen introduction (X-C from other birds; ice) | Trim to remove contamination from foreign matter. Proper cleaning of equipment and utensils (SSOP 3). |  |  | ##### YES | CCP #1 |
|  | Physical: none |  |  |  |  |  |
|  | ## Chemical: none |  |  |  |  |  |
| ##### Chill Carcass, Giblets and Neck | Biological: pathogen growth (X-C from other birds; ice) | Reduce temperature rapidly using ice water slurry. Temperature control (SSOP 5). Use ice made from potable water source (GMP 6). |  |  | YES | CCP #2 |
|  | Physical: none |  |  |  |  |  |
|  | Chemical: none |  |  |  |  |  |
| Drain Carcass, Giblets and Neck | Biological: pathogen introduction | Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). |  |  | NO |  |
|  | Physical: contamination from foreign matter | Proper cleaning of equipment and food contact surfaces (SSOP 3). |  |  |  |  |
|  | Chemical: none |  |  |  |  |  |
| Package, Weigh and Label | Biological: pathogen introduction (salmonella ssp.) from birds or infected personnel | Include proper cooking instructions on every food label (see MPPU Food Product Description). |  |  |  |  |
|  | Physical: contamination from foreign matter Chemical: none | Wash or trim to remove contamination from foreign matter (Final Inspection/CL 1). |  |  |  |  |

4.3 - Identifying Critical Limits, Monitoring and Corrective Actions

| Process/Step CCP |  | Critical Limit (CL) |  | Monitoring Procedures |  | Corrective Action (CA) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Final Inspection CCP #1 |  | No visible foreign matter. Zero tolerance for fecal matter and ingesta. |  | What will be measured? At least 2 % of birds, (5 minimum) will be monitored for presence of foreign matter and/or ingesta after final rinse and prior to chilling. Where will the CL be measured? Final inspection in the evisceration area. How will the CL be measured? Visual inspection. Who will monitor the CL? HACCP-trained grower-processor or designee. #### Frequency? For 200 birds or less: 5-bird sample over day. |  | How will the process be corrected? Trim away any foreign matter. ##### Product Disposition? Discard trimmings into container for inedibles. Who is responsible for implementing the CA? HACCP-trained grower-processor or designee. Measures to prevent any recurrence? Retrain personnel as needed. Adjust process as needed. |  |  |
| Chilling CCP #2 |  | ## Internal bird temperature <41F. |  | What will be measured? Internal temperature. Where will CL be measured? In the carcass cavity. How will CL be measured? Thermal probe. Who will monitor CL? HACCP-trained grower-processor or designee ##### Frequency? For 200 birds or less: 5-bird sample/day. |  | How will the process be corrected? Keep chilling until temperature is reached. ##### Product disposition? Reject (discard), chill or freeze. Who is responsible for implementing the CA? HACCP-trained grower-processor or designee. Measures to prevent recurrence? Retrain personnel. Adjust process. |  |  |
| ls (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). | NO |  |  |  |  |  |  |  |
|  | Physical: none Chemical: none |  |  |  |  |  |  |  |
| Trim Carcass/ Final Rinse (Inside and Outside) | Biological: pathogen Introduction Physical: none Chemical: none |  | Trim to remove any foreign matter that may cause contamination. Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). |  | NO |  |  |  |

| Process Step | Potential Hazard |  | What control measures can be applied to prevent the hazard? | Is potential safety hazard significant and reasonably likely to occur? |  | CCP # |
| --- | --- | --- | --- | --- | --- | --- |
| Final Inspection: Carcass, Giblets and Neck | Biological: pathogen introduction (X-C from other birds; ice) | Trim to remove contamination from foreign matter. Proper cleaning of equipment and utensils (SSOP 3). |  |  | ##### YES | CCP #1 |
|  | Physical: none |  |  |  |  |  |
|  | ## Chemical: none |  |  |  |  |  |
| ##### Chill Carcass, Giblet sand Neck | Biological: pathogen growth (X-C from other birds; ice) | Reduce temperature rapidly using ice water slurry. Temperature control (SSOP 5). Use ice made from potable water source (GMP 6). |  |  | YES | CCP #2 |
|  | Physical: none |  |  |  |  |  |
|  | Chemical: none |  |  |  |  |  |
| Drain Carcass, Giblets and Neck | Biological: pathogen introduction | Proper cleaning of equipment and utensils (SSOP 3). Proper personnel hygiene (GMP 2; SSOP 2). |  |  | NO |  |
|  | Physical: contamination from foreign matter | Proper cleaning of equipment and food contact surfaces (SSOP 3). |  |  |  |  |
|  | Chemical: none |  |  |  |  |  |
| Package, Weigh and Label | Biological: pathogen introduction (salmonella ssp.) from birds or infected personnel | Include proper cooking instructions on every food label (see MPPU Food Product Description). |  |  |  |  |
|  | Physical: contamination from foreign matter Chemical: none | Wash or trim to remove contamination from foreign matter (Final Inspection/CL 1). |  |  |  |  |

4.3 - Identifying Critical Limits, Monitoring and Corrective Actions

| Process/Step CCP | Critical Limit (CL) | Monitoring Procedures | Corrective Action (CA) |
| --- | --- | --- | --- |
| ##### Final Inspection CCP #1 | No visible foreign matter. Zero tolerance for fecal matter and ingesta. | What will be measured? At least 2 % of birds, (5 minimum) will be monitored for presence of foreign matter and/or ingesta after final rinse and prior to chilling. Where will the CL be measured? Final inspection in the evisceration area. How will the CL be measured? Visual inspection. Who will monitor the CL? HACCP-trained grower-processor or designee. #### Frequency? For 200 birds or less: 5-bird sample over day. | How will the process be corrected? Trim away any foreign matter. ##### Product Disposition? Discard trimmings into container for inedibles. Who is responsible for implementing the CA? HACCP-trained grower-processor or designee. Measures to prevent any recurrence? Retrain personnel as needed. Adjust process as needed. |
| Chilling CCP #2 | ## Internal bird temperature <41 F. | What will be measured? Internal temperature. Where will CL be measured? In the carcass cavity. How will CL be measured? Thermal probe. Who will monitor CL? HACCP-trained grower-processor or designee ##### Frequency? For 200 birds or less: 5-bird sample / day. | How will the process be corrected? Keep chilling until temperature is reached. ##### Product disposition? Reject (discard), chill or freeze. Who is responsible for implementing the CA? HACCP-trained grower-processor or designee. Measures to prevent recurrence? Retrain personnel. Adjust process. |

4.4 - HACCP Record Keeping and Verification Procedures

| Process/Step CCP | Records | Responsibility | CCP Verification |
| --- | --- | --- | --- |
| ##### Final Inspection CCP #1 | HACCP Final Inspection Logs (Carcass and Giblets) | ###### HACCP-trained grower-processor or designee | HACCP Final Inspection Logs will be verified each day of use by grower-processor or designee. |
| ##### Chilling CCP #2 | HACCP Temperature Log | HACCP-trained grower-processor or designee | HACCP Temperature Logs will be verified each day of use by grower-processor or designee. Personnel will be retrained each year by grower-processor or designee. |

Appendix A

Sample Operation Recordkeeping Logs

Sample MPPU Personnel Training and Observation Logs:

Safe Food Handling Plan Overview

Personnel Health and Hygiene Assessment

Sanitation Standard Operating Procedures

Processing Waste Management

On-Farm and Farm-to-Farm Bio-Security

Safe and Humane Poultry Processing and Handling;

Meeting Organic and Humane Livestock Handling Standards

MPPU Equipment Operation, Maintenance and Repair

Sample MPPU Daily Use Report Logs:

Monthly Log: Site Inspection and Pest Control

Daily Log: Personnel Health and Hygiene Assessment

Daily Log: Pre- and Post-Operational Inspection and Sanitation

Daily Log: Operational Sanitation Maintenance

Daily Log: Chill Tank/Refrigeration Temperature Monitoring

Daily Log: HACCP Log: Carcass and Giblets Final Inspection

Daily Log: HACCP Log: Carcass and Giblets Internal Temperature

Monitoring

Daily Log: Farm-to-Farm Bio-Security Protocol and Practices

Daily Log: MPPU Processing Water and Solid Waste Management Protocol and Practices and Appendix A

Sample MPPU Personnel Training and Observation Log: Safe Food Handling Plan Overview (Use to document GMP 1)

| # Training Date | ## Training Topic | ## Trainee(s) | ## Trained and Observed by: |
| --- | --- | --- | --- |

Training Topic: MPPU Safe Food Handling Plan, Review and Q &A

Signed/Date___________________________________________

Sample MPPU Personnel Training and Observation Log: Personal Health and Hygiene Practices (Use to document GMP 1)

| # Training Date | ## Training Topic | ## Trainee(s) | ## Trained and Observed by: |
| --- | --- | --- | --- |

Training Topics (Principles and Practice – Review of Personnel Hygiene Policies):

Personal Health and Food Safety

Personal Hygiene and Food Safety

Hygienic Hand Practices

Appropriate Work Attire

Hygiene and SSOP/HACCP (Focus on SSOP 1 and HACCP Analysis)

Signed/Date___________________________________________

Sample MPPU Personnel Training and Observation Log: Sanitation Standard Operating Procedures

(Use to document GMP 1)

| # Training Date | ## Training Topic | ## Trainee(s) | ## Trained and Observed by: |
| --- | --- | --- | --- |

Training Topics (Principles and Practice):

SSOP 2: Personnel Hygiene Assessment

SSOP 3: Managing and Documenting the Pre-Operational Inspection and Sanitation Schedule

SSOP 4: Daily Operational Sanitation Maintenance and Documentation

SSOP 5: Chill Tank and Refrigeration Temperature Monitoring

SSOP 6: Managing and Documenting the Post-Operational Sanitation Schedule		Signed/Date: ___________________________

Sample MPPU Personnel Training and Observation Log: On-Farm and Farm-to-Farm Bio-Security

(Use to document GMP 1)

| # Training Date | ## Training Topic | ## Trainee(s) | ## Trained and Observed by: |
| --- | --- | --- | --- |

Training Topic: (Principles and Practice):

Include MPPU Bio-Security Protocol as text.						Signed/Date___________________________________________

Sample MPPU Personnel Training and Observation Log: Processing Waste Management (Use to document GMP 1)

| # Training Date | ## Training Topic | ## Trainee(s) | ## Trained and Observed by: |
| --- | --- | --- | --- |

Training Topics (Principles and Practice)

Water Quality Protection; HEL Assessment; Environmentally Responsible Irrigation Practices

On-Farm Composting

Signed/Date___________________________________________

Sample MPPU Personnel Training and Observation Log: Safe and Humane Poultry Processing and Handling; Meeting Organic and Humane Livestock Handling Standards  (Use to document GMP 1)

| # Training Date | ## Training Topic | ## Trainee(s) | ## Trained and Observed by: |
| --- | --- | --- | --- |

Training Topics (Principles and Practice)

Include MPPU Operator’s Guide: Safe and Humane Poultry Processing and Handling, and Meeting Organic and Humane Livestock Handling Standards as text.

Signed/Date___________________________________________

Sample MPPU Personnel Training and Observation Log: Equipment Operation, Maintenance and Repair

(Use to document GMP 1)

| # Training Date | ## Training Topic | ## Trainee(s) | ## Trained and Observed by: |
| --- | --- | --- | --- |

Training Topic: (Principles and Practice):

Include MPPU Bio-Security Protocol as text.

Signed/Date___________________________________________

Sample Monthly Log: Farm Site Inspection and Pest Control

(Use to document SOP 1)

| # DATE (Month/ Year) | ## AREA INSPECTED/ ## CORRECTIVE ACTIONS NEEDED (IF ANY): | ## Initial/ ## Date: | ## NOTES/CORRECTIVE ## ACTIONS TAKEN: | Initial/ Date: |
| --- | --- | --- | --- | --- |
|  | 1 |  |  |  |
|  | 2 |  |  |  |
|  | 3 |  |  |  |
|  | 4 |  |  |  |
|  | 5 |  |  |  |
|  | 6 |  |  |  |
|  | 7 |  |  |  |
|  | 8 |  |  |  |

Once each month of permit duration:

            - Producer-processor visually inspects processing environment (grounds and buildings, including storage areas and sanitary facilities) for cleanliness and presence of pests and trash, once each month during operational period.

Lists needed corrective action and documents (initials log).

            - Producer-processor performs corrective actions and documents (initials log).

            - Producer-processor verifies, signs and dates each monthly log.

Signed/Date___________________________________________

Sample Daily Log: Personnel Health and Hygiene Assessment

(Use to document SSOP 2)

Farm:

Date:

Time:

| ## NAME: | ## ASSESSSED BY: | ## PASS/FAIL, COMMENTS | INITIAL/DATE (Both parties) |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |

Use additional sheets if required.

Once each day of operation, personnel checks for following and initials log:

Presence of food borne illness, and symptoms of a stomach or intestinal illness

Sore throat or temperature

Infected wounds or cuts

Household member with person who is ill

Personal cleanliness (hair, work clothes, shoes)

Presence of jewelry; need for hair or beard restraint

Working knowledge of proper hygienic hand practices

Producer-processor verifies, signs and dates.

Signed/Date___________________________________________

Sample Daily Log: Pre- and Post-Operational Inspection and Sanitation

(Use to document SSOPs 3 and 6)

Farm:

Date:

|  | PRE-OP INSPECTION/ CLEAN-UP: (Initial) | CLEAN/RINSE/ SANITIZE: (Initial) | POST OP INSPECTION/ CLEAN-UP/STORAGE: (Initial) | NOTES/CORRECTIVE ACTIONS REQUIRED and COMPLETED |
| --- | --- | --- | --- | --- |
|  |  | Pre-Op Post-Op |  |  |
| Killing cones |  | / |  |  |
| Scalder and plucker |  | / |  |  |
| Knives, implements and utensils |  | / |  |  |
| Evisceration and work tables |  | / |  |  |
| Chilling and holding tanks, tubs, etc. |  | / |  |  |
| Cleaning and sanitizing equipment |  | / |  |  |
| Pipes; hoses; water, propane and electric systems, backflow devices; floor, etc. |  | / |  |  |
| Sanitary facilities |  | / |  |  |

For each day of use, both before (pre-operation) and after (post-operation) use:

- Personnel visually inspect all water, electric and propane systems, and all processing equipment utensils for cleanliness and operability, and documents (initial log). Post-operation: picks up feathers and other matter, and  removes receptacles for inedible material and trash. Document.

- Personnel clean, rinse and sanitize all product contact surfaces, equipment and utensils. Repeats if necessary. Post-operation: applies edible oil to all surfaces subject to corrosion. Stores supplies. Document.

- Producer-processor verifies, signs and dates.							Signed/Date:__________________________________________

Sample Daily Log: Operational Sanitation Maintenance

(Use to document SSOP 4)

Farm:

Date:

Time:

| POTENTIAL HAZARD/EVENT* (IF ANY) | CORRECTIVE ACTION** REQUIRED and COMPLETED | SIGN and DATE |
| --- | --- | --- |

Examples:

- Hazard: carcass falls to the floor.

Corrective action: immediately pick up carcass and wash/rinse thoroughly before further processing. Document in Log.

- Hazard: poultry intestines are nicked during evisceration, contaminating evisceration table and utensils with fecal matter.

Corrective action: wash, rinse and sanitize processing area and utensils. Document in Log.

- Hazard: area of unit or piece of equipment becomes contaminated.

- **Corrective Action: Clean, rinse and sanitize, as per Pre-Operational Sanitation Procedures. Maintain clean and sanitary conditions throughout the daily operation. Document corrective action in Log.

Producer-processor verifies, signs and dates.

Signed/Date___________________________________________

Sample Daily Log: Chill Tank and Refrigeration Temperature Monitoring

(Use to document SSOP 5)

Farm:

Date:

Time:

| CHILL TANKS | TIME | TEMP. | CORRECTIVE ACTIONS | SIGNED |  | REFRIG. | DATE | TEMP. | CORRECTIVE ACTIONS | SIGNED |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | COOLER | DATE | TEMP. | START/END |  |
|  |  |  |  |  |  | 1 |  |  |  |  |
|  |  |  |  |  |  | 2 |  |  |  |  |
|  |  |  |  |  |  | 3 |  |  |  |  |

    - Personnel use digital thermometer to test temperature of chill tanks once each hour of operation. Maintain temperature between 33°F and 40°F.

    - Personnel use min-max thermometer to test temperature of refrigerator used to hold fresh poultry once each day in use. Maintain temperature at <40°F.

    - Personnel monitor Farmers’ Market cooler temperatures at 33°F-40°F. Add ice as required. Document temperatures at start and end of day.

    - Producer-processor verifies, signs and dates.

Signed/Date___________________________________________

Sample Daily HACCP Log: Poultry Carcass and Giblets Final Inspection

(Use to document CCP #1, Inspection)

Farm:

Date:

Time:

| Sample | Inspected by: | Pass | Fail | Corrective Action * | Re-inspected by: | Pass | Fail** | Signature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |

NOTE: 2% of product must be tested and documented. For 200 or fewer birds, a sample of at least five is required. Use additional forms if required.

Critical Control is necessary to reduce a biological hazard – the rapid growth of pathogens introduced by physical presence of fecal matter or ingesta.

* Corrective Action: Trim to remove contamination from foreign matter.

**Critical Limit: Zero tolerance for fecal matter and ingesta. No visible contamination.

        - Personnel manage final inspection of 2% (minimum of 5) product samples and document.

        - Producer-processor verifies, signs and dates.

Signed/Date___________________________________________

Sample Daily HACCP Log: Poultry Carcass and Giblets Internal Temperature Monitoring

(Use to document CCP #2, Chilling)

Farm:

Date:

Time:

| Sample | Test #1 | Tested by: | Pass | Fail | Corrective Action * | Test #2 | Tested by: | Pass | Fail** | Signature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |

NOTE: 2% of birds must be tested. For 200 or fewer birds, a sample of at least five is required. Use additional forms if required.

Critical Control is necessary to reduce a biological hazard – the rapid growth of pathogens introduced by X-C from other birds or from contaminated water or ice.

*Corrective Action: Keep chilling until critical limit is reached. Monitor chill tank temperature (at least once/hour) and document. The target temperature for chill      tank slurry is between 33°F and 40F. Add ice as necessary. See SSOP 5 for Chill Tank and Refrigeration Temperature Monitoring.

**Critical Limit: Internal carcass temperature <40F within four hours. If critical limit is not reached, product may not enter commerce. Cook or freeze immediately for personal use or discard.

1. Personnel monitor and document product internal temperature and chill tank temperature.

2. Producer-processor verifies, signs and dates.

Signed/Date___________________________________________

Appendix B

Sample Farm-to-Farm Bio-Security Protocol and Practices

(Use log to document Good Manufacturing Practices 3, 4 and 5

## Introduction

“Bio-security means doing everything you can to protect your birds from disease. As a [producer-processor], keeping your birds healthy is a top priority. Your birds can become sick or die from exposure to just a few unseen bacteria, viruses, or parasites. In a single day, these germs can multiply and infect all your birds….”

This quote, and most of the following information, is taken from the USDA Animal and Plant Health Inspection Service (APFIS) brochure: Backyard Bio-security Practices to Keep Your Birds Healthy.

Visit www.aphis.usda.gov/animal_health/ and www.ma.gov/agr to learn more.

APHIS offers several “Bio-security Tips” to prevent poultry disease. They are as effective for keeping the processing environment clean and sanitary as for use in the poultry-rearing areas. They include:

- Keep Your Distance. Restrict access to birds. Allow only people who care for them to come in contact with them. Keep a clean buffer zone around the area where they are housed or grazed.

- Keep it Clean. Set aside work clothes and shoes that are worn around your birds. Shoes can be scrubbed with a long-handled scrub brush dipped in a solution of household bleach (sodium hypochlorite 6 percent. Mix ¾ cup per gallon of water.) Consider installing a pail and brush near both entrances of the MPPU.

- Don’t Haul Disease Home. Car and truck tires, poultry cages and equipment can harbor “germs.” Be sure to disinfect these items – including tires of the MPPU and the truck that transports it -- before allowing them on the property. Scrub them with disinfectant and rinse at the entrance of the farm.

- Don’t Borrow Disease from a Neighbor. Do not share birds, equipment, tools or poultry supplies with other bird owners. Clean and disinfect any items before bringing them onto the property – and clean and disinfect before returning them. Never share items such as wooden pallets [or other items that are porous] and cannot be adequately cleaned and sanitized.

- Cleaning and disinfecting is one of the most important bio-security practices. Items must thoroughly cleaned and scrubbed with detergents before applying disinfectants or sanitizers. Sanitizers do not work on top of caked-on dirt. Rinse items well before applying a sanitizer with a brush, sponge or spray and allow adequate contact time.

#     

# Sample MPPU Farm-to-Farm Bio-Security Practices Log

FARM:

DATE:

- Processing personnel who raise poultry have changed into clean work clothing and shoes (or disinfected shoes) before entering this farm.

- Car, truck and MPPU trailer tires have been properly disinfected before being allowed onto this farm.

- Equipment, tools and/or supplies borrowed from or shared with other poultry producers, if any, have been carefully cleaned and sanitized before being brought onto this farm.

The MPPU Farm-to-Farm Bio-Security procedures described above were implemented on the above farm on this date.

Signed: ________________________________________

Producer-processor			 		MPPU Use Date

Appendix C

Sample MPPU Processing Water and Solid Waste Management Protocol and Practices Log

Good Manufacturing Practice 9 (see also, Appendix A to this document)

A. Processing Location: The MPPU unit was parked on (check one):

_____ 	biologically active grass, farm hayfields or pasture.

_____	an agricultural compost pad provided with a biologically active buffer strip, located at   least 100’ from active cropland.

B. Water:

_____ Water from pre- and post-process cleaning/rinsing and hand sinks not contained (i.e., was allowed to flow directly to an actively growing grassed area), was discharged in a manner that would not cause erosion or impact surface water, groundwater or other resource areas. No harsh cleaning and disinfectant chemicals were introduced into such rinse water; prior to commencement of post-process rinse, all solid waste (e.g., viscera, offal, feathers) was physically removed from equipment and food contact surfaces.

_____ Water generated during processing (i.e., from scalder, plucker and evisceration tables, or from initial cleaning of contact surfaces that contain solids) was incorporated into a compost pile.

Water contained in chill tanks was (check one):

_____ 	applied to actively growing agricultural land verified by MDAR during site inspection as adequate to accept this application; or

_____ 	discharged directly into a compost pile (see B, below). Such discharge shall not cause the water to migrate beyond the compost pile boundary.

Completed by: _________________________________________

Signed: _______________________________________________

Producer-processor			 		MPPU Use Date

C.	Solid Waste: Solid waste, e.g., feathers, blood, viscera and inedible processing byproducts have been properly collected and disposed of as follows (check one):

_____	Placed in an on-site dumpster with regularly scheduled pick-up for transport to a licensed solid waste facility.

_____	Incorporated into an approved active or newly constructed agricultural compost pile (minimum 4’wide x 5’high, and 6’long per 100 birds). Also see Appendix A to this document.

Completed by: _________________________________________

Signed: _______________________________________________

Producer-processor			 		MPPU Use Date

D. Trash: All trash generated by poultry processing activity on this date, e.g., paper towels, discarded containers or packaging materials, and disposable gloves, has been properly collected, contained and removed from the processing environment.

Completed by: ________________________________________

Signed: ______________________________________________

Producer-processor			 		MPPU Use Date

Appendix D

DEP Guidelines and Best Management Practices for MPPU Waste Management